### Working Group Report

The reports should cover issues raised by pilots, organisers, SCC members, delegates, Steward & Jury reports etc.; Outcomes (recommendations, proposals, actions); and concerns.

| Subcommittee or Working Group: Altitude Working Group | Date: 03.01.10 |
| --- | --- |
| Name: Agust Gudmundsson chairman Brian Harris report writer. | Confidential to Bureau: N For publication: Y |

Summary of subcommittee activity since the last report: Since the 2009 Plenary, the WG has had a detailed discussion on the issues that occur when trying to measure and verify altitude in competitions. The discussion focused on the differences between using GPS altitude records and barometric based records stored in track logs, and also the lack of consistency between different instruments from various manufacturers. A number of documents were used as a reference to discussing the pros and cons of both methods. The general consensus of the group was that in the short term we should be basing altitude control on barometric measurements. This is in line with the reference used by other general aviation users and the measurement used to delineate controlled air space. It was felt that using GPS altitude was still subject to errors, and not easily compatible with airspace restrictions. It was noted that Garmin GPS units with pressure sensors when set in auto calibrate mode use GPS altitude to update barometric altitude, and it is this somewhat unquantifiable reading that is stored in the track. In addition these instruments can be recalibrated in flight, which could give the appearance of having avoided air space. The consensus was that these instruments are not suitable for CAT 1 competition. It was noted that the PWCA continues to use GPS Altitude for control in World Cup competitions. A document was produced to provide a method of consistently verifying altitude during the HG World Championships 2009. Due to the imminent date of the event it was felt that it would be unfair to ban certain instruments. This was approved by the CIVL bureau as an addendum to local regulations. This document allowed scorers to verify altitude using standard barometric pressure but also to check GPS-only instruments, using a known difference between a barometric value and a GPS value taken from the same track. This procedure was used in the Championship and allowed the scorers to deal with altitude violations. It was noted that the difference found between barometric and GPS values was about 4 or 5%, basically depending on pressure, and temperature of the day. The FS scoring system was enhanced by adding a facility for checking barometric altitude as well as GPS altitude. This is carried out in the FSFlight component and measured against values in a local open-air txt file. GPSdump was similarly enhanced this year to store both types of altitude records if present in IGC and kml files. There was very limited discussion of penalties for infringements. Generally it was felt these are the province of the PG and SC subcommittees. There was some limited contact with instrument manufacturers. In the case of Aircotec this resulted in GPS altitude being added to the tracklog.

Issues raised, discussed or actioned and resolved: Addendum produced for the 2009 HG Worlds. This has subsequently been added to Local Regulations in 2010 CAT 1 competitions, subject to ratification at the 2010 Plenary.

Issues raised to be resolved, suggested actions: Consider the implications of allowing Garmin instruments with pressure sensors (which can have their altitude track logs modified in flight) and allowing Garmin instruments that record interpolated/anticipated track log points, to continue to be allowed in CAT 1 competitions. Consider if the next step is to accept barometric only track logs in CAT 1 competitions with the standard pressure of 1013,2 hPa as is in IGC files. If so how should it be phased in. Review and discuss the feasibility of the use of emerging technology of live tracking system, such as the Safly DSX capable of producing IGC compliant tracklogs.

Issues or concerns requiring Plenary or Bureau guidance/response: Continue WG in 2010 SC or Plenary to review next step.

Issues of potential interest/relevance to other SCCs: Garmin with Pressure Sensors Garmin which interpolate/anticipate tracklog points. Live tracking systems.